# 📄 Secure Document Destruction Solution - for Professional Operators

## Integrated Shredding Lines - 2,000 Tons / Year

### 1. Project Overview

This solution is designed for professional document destruction operators providing outsourced services to banks and financial institutions.

The system focuses on long-term operational stability, security compliance, and cost efficiency, which are critical for service providers managing continuous workloads from multiple banking clients.

- Total annual volume: approx. 2,000 tons

  - Normal documents: 1,500 tons / year

  - High-security documents (EU P6–P7): 500 tons / year

- Operational assumption:

  - 200 working days per year

  - 6 operating hours per day

- Site condition:

  - Centralized operation at one facility

  - Available area ≥ 500 m², height approx. 6 m

- Design objective:

  - Stable, scalable operation for outsourced destruction services

  - Clear separation of security levels for different banking requirements

### 2. Overall System Concept

The proposed system is designed specifically for operators, not for one-time or internal bank use.

- Normal and high-security documents are processed on separate, dedicated lines

- Parallel line architecture ensures:

  - Continuous operation

  - Easy maintenance without full shutdown

  - Lower operational risk for service providers

- The system is suitable for multi-client, long-term service contracts

## 3. Line A – Normal Document Shredding (EU P2–P3)

Annual volume: approx. 1,500 tons

- Required throughput: 1.2–1.3 tons/hour

- Configuration: 2 parallel shredding lines

Each line includes:

- Primary shredder

  - Motor power: 15 kW

- Secondary shredder

  - Motor power: 10 kW

  - Security level: EU P2–P3

- Single-line capacity: 600–700 kg/hour

Discharge & baling:

- Shredded paper is sent directly to the customer’s existing baler

- Large paper bales for easy handling and resale

- No additional baler investment required

## 4. Line B – High-Security Document Shredding (EU P6–P7)

Annual volume: approx. 500 tons

- Required throughput: approx. 400–420 kg/hour

- Configuration: 2 parallel high-security shredding lines

Each line includes:

- Conveyor feeding system

- Primary shredder

  - Motor power: 11 kW

- High-security fine shredder

  - Motor power: 22 kW

  - Output size: 1–2 mm particles

  - Security level: EU P6–P7

- Single-line capacity: 150–200 kg/hour

Final treatment:

- Shredded paper powder is processed by a hydraulic briquetting system

- Compressed into small cylindrical briquettes

## 5. Key Advantages for Operators

- ✔ Designed for outsourced document destruction services

- ✔ Parallel production lines ensure high uptime and operational flexibility

- ✔ Separation of security levels improves workflow efficiency

- ✔ Industrial-grade design suitable for continuous daily operation

- ✔ Optimized investment structure and long-term ROI for operators

This solution concept is intended for professional operators serving banks, central banks, and financial institutions.

### 🌐 Contact Us 联系我们

Shanghai YUEDU Technology Co., Ltd. | 上海越都科技有限公司

📧 Email: yuedushredders@gmail.com | denghao8410@gmail.com 
📱 WhatsApp / WeChat: +86 159 0178 2779
🌐 Website: www.yuedushredders.com

@YUEDUShredders – Empowering Global Industries with Intelligent Destruction Systems.

📄 银行档案文件保密销毁整体解决方案
面向专业运营方的集成式粉碎生产线方案
处理能力：2,000 吨 / 年

## 1. 项目概述（Project Overview）

本解决方案专为专业文件销毁运营方（Document Destruction Operators）设计，用于为银行及金融机构提供外包式档案销毁服务。

系统设计重点放在长期运行稳定性、保密合规性以及整体成本效率上，适用于需要同时服务多家银行客户、长期连续运行的运营场景。

- 年总处理量：约 2,000 吨

  - 普通文件：1,500 吨 / 年

  - 高保密文件（EU P6–P7）：500 吨 / 年

- 运行假设：

  - 每年 200 个工作日

  - 每天 6 小时 实际运行时间

- 场地条件：

  - 单一厂区集中运营

  - 可用面积 ≥ 500 平方米

  - 厂房高度约 6 米

- 设计目标：

  - 适用于外包销毁服务的稳定、可扩展运营

  - 不同保密资料对应不同保密等级，清晰分线处理

## 2. 整体系统设计理念（Overall System Concept）

该系统是从运营方视角出发设计的工业级方案，并非一次性项目或银行内部自用系统。

- 普通文件与高保密文件 分别在独立生产线上处理

- 并行生产线架构可实现：

  - 连续稳定运行

  - 单线维护时无需全线停机

  - 降低运营方整体运行风险

- 系统非常适合：

  - 多客户并行

  - 长期服务合同

  - 规模化档案销毁运营

## 3. Line A —— 普通文件粉碎线（EU P2–P3）

年处理量：约 1,500 吨

- 目标产能：约 1.2–1.3 吨 / 小时

- 配置方式：2 条并行粉碎生产线

### 每条生产线配置包括：

- 一级粉碎机（Primary Shredder）

  - 电机功率：15 kW

- 二级粉碎机（Secondary Shredder）

  - 电机功率：10 kW

  - 保密等级：EU P2–P3

- 单线产能：约 600–700 kg / 小时

### 出料与打包方式：

- 粉碎后的纸张 直接进入客户现有打包机

- 打包成大尺寸纸包，便于搬运、存储及再销售

- 无需额外投资新的打包设备

## 4. Line B —— 高保密文件粉碎线（EU P6–P7）

年处理量：约 500 吨

- 目标产能：约 400–420 kg / 小时

- 配置方式：2 条并行高保密粉碎生产线

### 每条生产线配置包括：

- 输送带自动上料系统

- 一级粉碎机

  - 电机功率：11 kW

- 高保密细碎机

  - 电机功率：22 kW

  - 出料尺寸：1–2 mm 粉末颗粒

  - 保密等级：EU P6–P7

- 单线产能：约 150–200 kg / 小时

### 末端收集和压块：

- 粉碎后的纸粉进入液压式纸粉压块系统

- 压缩成小型圆柱状压块

## 5. 运营方核心优势总结（Key Advantages for Operators）

- ✔ 专为外包式文件销毁运营模式设计

- ✔ 并行生产线保障高开机率与运营灵活性

- ✔ 不同保密等级分线处理，提升整体流程效率

- ✔ 工业级设计，适合长期、连续日常运行

- ✔ 投资结构合理，有利于运营方长期 ROI

本方案适用于为银行、中央银行及金融机构提供服务的专业运营方。

📞 Contact Us

Johnny Deng, Managing Director

☎️  Phone: +86 159 0178 2779 (WhatsApp / WeChat / Facebook / Telegram)

📧Email: denghao8410@gmail.com

Shanghai YUEDU Technology Co., Ltd. (YUEDU Shredders)

🌐 Website(Global): www.yuedushredders.com (note: VPN required for China access, as the server is hosted by Google.)

📧 Email: yuedushredders@gmail.com / denghao8410@gmail.com

☎️  Sales Supervisor: +86 159 0178 2779  (WhatsApp / WeChat / Facebook / Telegram) ☎️  Administrative Supervisor: +86 137 6138 5116

☎️  Head Office Main Line: +86 21 519 000 20

📍 Company Headquarters: Building 12, No. 555 Chengyin Road, Baoshan City Industrial Park, Shanghai, China

与我们联系（中文普通话）

上海越都科技有限公司

📍公司总部：上海市宝山城市工业园区城银路555弄12号楼

☎️  中文电话: 138 1115 0898（同微信）, 137 1700 3125（同微信）, 188 1682 6112（同微信）

☎️  公司总机：021 - 519 000 20

☎️  行政总裁：137 6138 5116

☎️  投诉电话：159 0178 2779

📧  Email邮箱: denghao8410@gmail.com

🌐 Website(中文网站): www.91suizhiji.com

🌐 文档下载站： www.daxingsuizhiji.com